A Cyber-Physical Anomaly Detection for Wide-Area Protection using Machine Learning

ABSTRACT

Wide-area protection scheme (WAPS) provides system-wide protection by detecting and mitigating small and large-scale disturbances that are difficult to resolve using local protection schemes. As this protection scheme is evolving from a substation-based distributed remedial action scheme (DRAS) to the control center-based centralized RAS (CRAS), it presents severe challenges to their cybersecurity because of its heavy reliance on an insecure grid communication, and its compromise would lead to system failure.This paper presents an architecture and methodology for developing a cyber-physical anomaly detection system (CPADS) that utilizes synchrophasor measurements and properties of network packets to detect data integrity and communication failure attacks on measurement and control signals in CRAS.The proposed machine leaning-based methodology applies a rules-based approach to select relevant input features, utilizes variational mode decomposition (VMD) and decision tree (DT) algorithms to develop multiple classification models, and performs final event identification using a rules-based decision logic. We have evaluated the proposed methodology of CPADS using the IEEE 39 bus system for several performance measures (accuracy, recall, precision, and F-measure) in a cyber-physical testbed environment. Our experimental results reveal that the proposed algorithm (VMD-DT) of CPADS outperforms the existing machine learning classifiers during noisy and noise-free measurements while incurring an acceptable processing overhead.

CHAPTER 2

2. SYSTEM ANALYSIS

  - EXISTING SYSTEM

- In the case of cyber-attacks against a power system, human judgment is less certain since there is an overt attempt to disguise the attack and deceive the operators as to the true state of the system.

- To enable the human decision maker, we explore the viability of machine learning as a means for discriminating types of power system disturbances, and focus specifically on detecting cyber-attacks where deception is a core tenet of the event.

- We evaluate various machine learning methods as disturbance discriminators and discuss the practical implications for deploying machine learning systems as an enhancement to existing power system architectures.

2.1.1 DRAWBACKS OF EXISTING SYSTEM  :

- They have developed a system to identify different types of anomalies in a distribution system.

- They used an artificial neural network based approach to detect abnormal operation behaviors in the system. Further, dimensional reduction techniques such as principal component analysis was used to decompose the real-time monitoring and control data.

- A future direction for this research is to explore classification schemes and learner configuration to more thoroughly address this issue, including the possibility of staging learners for optimum classification performance.

2.2. PROPOSED SYSTEM

- The Chee-Wooi Ten IDS is host-based thus only identifies attacks against a single IED in the substation using sequential events recorded in the log from that IED.

- Another IDS proposed by Chen et al. in provides a protection mechanism for smart household appliances.

- Chen et al. created security rules for individual appliances by proposing homogeneous functions that models three factors of the appliance: device security, usability and electricity pricing.

- More advanced IDS of this type will consider behaviors of multiple devices within the system to obtain system level detection

2.2.1. ADVANTAGES OF PROPOSED SYSTEM

- IREST used unsupervised learning for training the cyber and physical ML anomaly detection algorithms.

- The results showed that unsupervised learning provided comparable performance with respect to supervised approaches, with the added benefit that abnormal behavior data is not required for training.

- This is important as unsupervised learning offers several advantages over supervised methods.

- In our case, unsupervised learning is especially useful as no attack data is needed for training.

Literature Survey:

| TITLE | AUTHORS | DESCRIPTION |
| --- | --- | --- |
| Framework for data driven health monitoring of cyberphysical systems | K. Amarasinghe, C. Wickramasinghe, D. Marino, C. Rieger, and M. Manicl, | This paper presents a framework for calculating and monitoring of health in CPSs using data driven techniques. |
| Cyber-physical systems: The next computing revolution | R. Rajkumar, I. Lee, L. Sha, and J. Stankovic, | Cyber-physical systems (CPS) are physical and engineered systems whose operations are monitored, coordinated, controlled and integrated by a computing and communication core. |
| Security management of cyber physical control systems using nist sp 800-82r2 | A. A. Jillepalli, F. T. Sheldon, D. C. de Leon, M. Haney, and R. K. Abercrombie, | In a previous paper, we described the Cyberspace Security Econometrics System (CSES), which is a stakeholder-aware and economics-based risk assessment method for cybersecurity. |
| Cyber-physical security testbeds: Architecture, application, and evaluation for smart grid | A. Hahn, A. Ashok, S. Sridhar, and M. Govindarasu, | The development of a smarter electric grid will depend on increased deployments of information and communication technology (ICT) to support novel communication and control functions. |

2.3 FEASIBILITY STUDY

The feasibility Analysis is an analytical program through project manager determines the project success ratio and through feasibility study project manager able to see either project. The key considerations involved in the feasibility analysis are:

- Economic Feasibility

- Technical Feasibility

- Operational Feasibility

- Environmental Feasibility

2.3.1 ECONOMICAL FEASIBILITY

Hence this project is economically feasible there is no need to involve any cost for this project.

2.3.2 TECHNICAL FEASIBILITY

Software Technologies used are PHP and MySql. In the educational institutions, it is possible to update the system in future. No special hardware is required for the purpose of using this system. Hence it is declared that this project is technically feasible.

2.3.3 OPERATIONAL FEASIBILITY

As the admin work mainly to maintain the Patient and Doctor .Doctor will predict patient cancer disease. Hence it is easy to operate with training. Therefore it is operationally feasible for implementation.

2.3.4 ENVIRONMENTAL FEASIBILITY

This project environment is correct as a admin has developed this system and no expenditure is involved under any head and this process is part of admin document management, this project environment is accessible.

2.4 SYSTEM REQUIREMENTS

- System	:  Pentium IV 2.4 GHz.

- Hard Disk	: 40 GB.

- Monitor	: 15 inch VGA Color.

- Mouse	: Logitech Mouse.

- Ram		: 512 MB

- Keyboard	: Standard Keyboard

2.4.1 HARDWARE REQUIREMENTS

The Hardware of the computer consists of physical component such as Input Devices, Storage Devices, Processing & Control units and Output Devices.  Computer includes external storage unit to store data in programs.

The Hardware Configuration involved in this project

- System	:  Pentium IV 2.4 GHz.

- Hard Disk	: 40 GB.

- Monitor	: 15 inch VGA Color.

- Mouse 		: Logitech Mouse.

- Ram		: 512 MB

- Keyboard	: Standard Keyboard

2.4.2 SOFTWARE REQUIREMENTS

Software is a group of programs that computers need to do a particular task.  It is an essential requirement of Computer System. The Software used to develop the project is

- Operating System	: Windows XP.

- Platform		: DOT NET TECHNOLOGY

- Front End		: ASP.Net 3.5

- Back End		: SQL SERVER 2005

###### CHAPTER 3

SYSTEM DESIGN AND DEVELOPEMENT

SYSTEM ARCHICTURE

CHAPTER 4

TESTING AND IMPLEMENTATION

  - TESTING

Implementation is the stage of the project when the theoretical design is turned into a working system. This is the final and important phase in the system life cycle It is actually the process of converting the new system into a operational one.

4.1.1Unit Testing

Unit testing comprises the set of tests performed by an individual programmer prior to integration of the unit into a larger system. The module interface is tested to ensure that information properly flows into and out of the program unit. The local data structure is examined to ensure that data stored temporarily maintains its integrity during all steps in an algorithm’s execution. Boundary conditions are tested to ensure that the module operates properly at boundaries established to limit or restrict processing. All independent paths through the control structure are tested. All error-handling paths are tested.

4.1.2 Block Box Testing

Black-box testing is a method of software testing that examines the functionality of an application without peering into its internal structures or workings. This method of test can be applied virtually to every level of software testing: unit, integration, system and acceptance. It is sometimes referred to as specification-based testing.

4.2 SYSTEM IMPLEMENTATION

Implementation is the stage of the project when the theoretical design is turned into a working system. This is the final and important phase in the system life cycle It is actually the process of converting the new system into a operational one.

CHAPTER 5

CONCLUSION:

In this paper we presented a Machine Learning approach for cyber and physical anomaly detection on Smart-Grid CyberPhysical Systems. We presented the IREST sensor which uses packet data to detect cyber and physical abnormal behavior. The sensor uses machine learning models to characterize the normal behavior of the system. IREST considers both cyber and physical data in order to construct a complete representation of a CPS. Data collection, testing and validation of the IREST sensor was performed on the ISAAC tested. IREST used unsupervised learning for training the cyber and physical ML anomaly detection algorithms. The results showed that unsupervised learning provided comparable performance with respect to supervised approaches, with the added benefit that abnormal behavior data is not required for training. Thanks to the success of the unsupervised methods, the IREST sensor is able to detect previously unseen cyber and physical anomalies.

CHAPTER-6

REFERENCE

- [1] K. Amarasinghe, C. Wickramasinghe, D. Marino, C. Rieger, and M. Manicl, “Framework for data driven health monitoring of cyberphysical systems,” in 2018 Resilience Week (RWS), Aug 2018, pp. 25– 30.

- [2] R. Rajkumar, I. Lee, L. Sha, and J. Stankovic, “Cyber-physical systems: The next computing revolution,” in Design Automation Conference, June 2010, pp. 731–736.

- [3] A. A. Jillepalli, F. T. Sheldon, D. C. de Leon, M. Haney, and R. K. Abercrombie, “Security management of cyber physical control systems using nist sp 800-82r2,” in 2017 13th International Wireless Communications and Mobile Computing Conference (IWCMC), June 2017, pp. 1864–1870.

- [4] A. Hahn, A. Ashok, S. Sridhar, and M. Govindarasu, “Cyber-physical security testbeds: Architecture, application, and evaluation for smart grid,” IEEE Transactions on Smart Grid, vol. 4, no. 2, pp. 847–855, 2013.

- [5] B. Bagheri, S. Yang, H.-A. Kao, and J. Lee, “Cyberphysical systems architecture for self-aware machines in industry 4.0 environment,” IFAC-PapersOnLine, vol. 48, no. 3, pp. 1622 – 1627, 2015, 15th IFAC Symposium onInformation Control Problems inManufacturing. [Online]. Available: http: //www.sciencedirect.com/science/article/pii/S2405896315005571

- [6] I. A. Oyewumi, A. A. Jillepalli, P. Richardson, M. Ashrafuzzaman, B. K. Johnson, Y. Chakhchoukh, M. A. Haney, F. T. Sheldon, and D. C. de Leon, “Isaac: The idaho cps smart grid cybersecurity testbed,” in 2019 IEEE Texas Power and Energy Conference (TPEC), Feb 2019, pp. 1–6.

- [7] I. A. Oyewumi, H. Challa, A. A. Jillepalli, P. Richardson, Y. Chakhchoukh, B. K. Johnson, D. Conte de Leon, F. T. Sheldon, and M. A. Haney, “Attack scenario-based validation of the idaho cps smart grid cybersecurity testbed (isaac),” in 2019 IEEE Texas Power and Energy Conference (TPEC), Feb 2019, pp. 1–6.

- [8] C. S. Wickramasinghe, D. L. Marino, K. Amarasinghe, and M. Manic, “Generalization of deep learning for cyber-physical system security: A survey,” in IECON 2018 - 44th Annual Conference of the IEEE Industrial Electronics Society, Oct 2018, pp. 745–751.

- [9] A. Jones, Z. Kong, and C. Belta, “Anomaly detection in cyber-physical systems: A formal methods approach,” in 53rd IEEE Conference on Decision and Control, 2014, pp. 848–853.

- [10] J. Goh, S. Adepu, M. Tan, and Z. S. Lee, “Anomaly detection in cyber physical systems using recurrent neural networks,” in 2017 IEEE 18th International Symposium on High Assurance Systems Engineering (HASE), 2017, pp. 140–145.